| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Forklift / Moving Operation No 1 A.M.K Electronics Park Rd |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 17th May 2017 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Unloading equipment from lorry chassis to loading bay | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working space is clear of people and any obstruction - Drive at slow transportation speed | Major | Remote | Medium | Assignment of traffic controller | Forklift Driver / Supervisor on site |
|  |  | Falling load | Bodily injury & damage to property | - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure load weight are spread evenly and is stable on the fork before driving off | Major | Remote | Medium | Use of extension fork if needed to give greater stability | Forklift Driver / Supervisor on site |
|  |  | Restricted driver visibility | Bodily injury, damage to site property | - Forklift equipped with side mirrors and reverse warning alarm to caution nearby people - Ensure forklift driver is guided by other staff - If required, reverse driving with reverse lights on - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Driver / Supervisor on site |
|  |  | Blind spots | Bodily injuries to workers and nearby people. | - Operator advise to be careful when moving forward and reversing - Press horn if necessary to warn others that forklift is approaching - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Driver / Supervisor on site |
| 2 | Transportation of equipment to designated location | Unstable load while moving | Bodily Injury & damage to property | - Ensure pallet truck are place accordingly on equipment base - Ensure equipment load are spread evenly and is stable on the pallet truck - Workers to be alert and keep close lookout on route of travel from point to point - Buddy system-one to handle the pallet truck steering while the others to push and hold on onto the equipment | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site on site |
|  |  | Passage way block by material/ tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | Moderate | Occasional | Medium | Supervisor to make sure route of transporting before moving and double check clearing of all material/ tooling | Supervisor on site on site |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close watch of travel - Ensure movement is guided - To alert workers controlling the pallet truck steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site on site |