| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Forklift / Lifting Operation 501 Airport Rd ST Aerospace Engines |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 18th Aug 2016 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Fork up machine | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working space is clear of people and any other obstruction - Drive at slow transportation speed | Major | Remote | Medium | Assignment of traffic controller | Forklift Driver / Supervisor on site |
|  |  | Falling load | Bodily injury & damage to property | - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure machine weight are spread evenly and is stable on the fork before driving off | Major | Remote | Medium | Use of extension fork if needed to give greater stability | Forklift Driver / Supervisor on site |
| 2 | Transport machine to loading bay | Unstable load while moving | Bodily injury & damage to property when load topple | - Ensure forklift fork are place accordingly on machine bed - Ensure load are spread evenly and is stable on the fork - Workers to be alert and keep close lookout on route of travel from point to point | Major | Remote | Medium | Slow and steady speed | Forklift Driver / Supervisor on site |
|  |  | Restricted driver visibility | Bodily injury & damage to site property | - Forklift equipped with side mirrors and reverse warning alarm to caution nearby people - Ensure forklift driver is guided by other staff - If required, reverse driving with reverse lights on - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Driver / Supervisor on site |
|  |  | Blind spots | Bodily injuries to workers and nearby people | - Operator advise to be careful when moving forward and reversing - Press horn if necessary to warn others that forklift is approaching - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Driver / Supervisor on site |
| 3 | Lifting operation | Failing of lifting equipment | Bodily injury & damage to property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe ) provided and put on | Major | Remote | Medium | While moving or slewing the crane, try not to jerk the load | Lifting supervisor on site |
|  |  | Overloading | Bodily injury & damage to property | - Ensure overloading alarm device is functioning - No lifting beyond SWL of crane - Load weight should be known | Major | Remote | Medium | Ensue load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | Lifting supervisor on site |
|  |  | Stuck by load due to unsafe slinging, rigging | Bodily injury & damage to property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out task - Load shall be securely slug, properly balance and trimmed to prevent from dislodgment - Ensure load are rig at it centre of gravity | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting supervisor on site |
|  |  | View obstruction, load swing | Bodily injury & damage to property | - Clear all obstruction and unauthorised personnel in way of lifting path - Use tag line for load when necessary to prevent of load swing - Never stand close or under the suspended load and away from possible swing path | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting supervisor on site |