| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Raja Zaharin (Team Member) |  |
| Process/ Location: | Loading/unloading of equipment |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 14 March 2016 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Loading / unloading by forklift | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure working space is clear of unauthorised personnel and any obstruction - Ensure forklift driver, if needed is guided by other staff - Drive at slow transportation speed | Major | Remote | Medium | Assignment of traffic controller | Forklift Driver / Supervisor |
|  |  | Falling load during loading or unloading | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift. - Known the CG of the load and understand the load chart of the cargo - Elevate the cargo slightly and fully tilt the mast backward before driving off - Ensure stability of cargo before driving off - No unauthorised personnel within the working zone | Major | Remote | Medium | Use extension fork, if needed, to give greater stability of cargo | Forklift Driver / Supervisor |
| 2 | Transporting of load to location | Restricted driver visibility | Bodily injury, damage to site property | - Forklift equipped with side mirrors and reverse warning alarm to caution nearby people. - Forklift driver to ensure that he turned his head to check whether anyone behind the forklift before proceeding to drive - Ensure forklift driver is guided by other staff if needed - If required, reverse driving with reverse lights on - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Driver / Supervisor |
|  |  | Blind spots | Bodily injuries to workers and nearby people. | - Operator advise to be careful when moving forward and reversing - Press horn if necessary to warn others that forklift is approaching | Moderate | Occasional | Low | Assignment of traffic controller | Forklift Driver / Supervisor |