| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin ( Team Member) |  |
| Process/ Location: | Forklift / Un-crate case / Moving Operation Blk 22 Pioneer Crescent #06-06 |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 08 Jan 2015 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2012 | Next Review Date: | 14 May 2015 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Loading / unloading by forklift | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure working space is clear of unauthorised personnel and any obstruction - Drive at slow transportation speed | Major | Remote | Medium | If necessary, deploy staff to play the role of traffic controller | Forklift Operator |
| 2 | Un-crate case | Protruding nails or other sharp objects | Bodily injury | - Keep hand clear of sharp edges - Remove sharp objects (etc…nails, metal strip) from working area as soon as possible - PPE (etc… safety boot, glove) provided and put on | Moderate | Remote | Low |  | Supervisor on site |
|  |  | Falling of wooden planks | Bodily injury | - Un-crate and dismantle case in the correct sequence - Remove immediately any loose planks - Good coordination between workers - Ensure no unauthorised personnel is nearby during un-crating work | Moderate | Remote | Low |  | Supervisor on site |
| 3 | Detach machine from base | Falling load during loading or unloading | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift. - Known the CG of the load and understand the load chart of the cargo - Ensure stability of cargo before driving off - Elevate the cargo slightly and fully tilt the mast backward before driving off - No unauthorised personnel in the working zone | Major | Remote | Medium | Use extension fork if needed, to give greater stability of cargo | Forklift Operator |
| 4 | Transporting machine to main entrance | Restricted driver visibility | Bodily injury, damage to site property | - Forklift equipped with side mirrors and reverse warning alarm to caution nearby people. - Forklift driver to ensure and check whether anyone behind or near the forklift before proceeding to drive - Ensure forklift driver is guided by other staff - Slow transportation speed | Moderate | Remote | Low | Assignment of traffic controller | Forklift Operator |
|  |  | Blind spots | Bodily injuries to workers and nearby people. | - Operator advise to be careful when moving forward and reversing - Press horn if necessary to warn others that forklift is approaching | Moderate | Remote | Low | Assignment of traffic controller | Forklift Operator |
| 5 | Using rollers/ pallet truck to move in machine into premise | Unstable/topper of load while moving | Bodily injury, damage to site property | - Ensure rollers/pallet truck are place accordingly on machine base - Ensure machine is stable on the rollers/pallet truck before being move to location - Workers to be alert and keep close lookout on route of travel from point to point | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | Moderate | Remote | Low | Close supervision by supervisor | Supervisor on site |
|  |  | Obstruction to vision | Damage to property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers/ pallet truck steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |