| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Shift & Position 1 unit Climbing Rock Stone to position |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 22 May 2015 |  |  |  |  |
|  |  | Last Review Date: | # 12 May 2015 | Next Review Date: | 11 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Insert rollers to stone base | Sharp edges of stone | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers to stone base with care - Use glove | Minor | Occasional | Low |  | Supervisor on site |
| 2 | Transportation of stone from loading area to location | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on stone base - Rollers are properly secured to stone base with bolts and nuts to prevent from rolling out - Ensure stone is stable on the rollers before being move to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Excessive weight load | Sprain Back | - Use the correct capacity rollers to reduce the effort of each man when pushing the stone - Deploy adequate personnel | Moderate | Occasional | Medium | Use machine puller or forklift if possible to assist in pulling/pushing the stone | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | Minor | Occasional | Low | Good coordination | Supervisor on site |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | -To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |
| 3 | Using ‘A’ frame structure with 10 ton chain block to load stone into hole | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Ensure lifting gears use are in compliance to weight of load - No defective gears allowed - PPE (etc…safety shoe, helmet) provided and put on | Moderate | Remote | Low |  | Supervisor on site |