| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd (System Manufacturing & Supplies) | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Relocation of Oven No 51 Calshot Rd (Eagle Services) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 30 May 2016 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Lift up oven using crowbar | Load topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure load are lift up at the right angle of gravity - Use the correct capacity crowbar to lift up the machine - Lift up load to 3” high from one side to another is carried out in stages with the use of hard wooden block | Major | Remote | Medium | Only skill and experience worker to control the crowbar handling | Supervisor on site |
| 2 | Insert rollers / pallet truck | Sharp edges of oven | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers/pallet truck to base of oven with care - Use glove | Minor | Occasional | Low |  | Supervisor on site |
| 3 | Transportation of machine to new location within premise | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers/pallet truck are place accordingly on oven base - Rollers are properly secured to oven base with bolts and nuts to prevent from rolling out - Ensure oven load are spread evenly on the rollers/pallet truck before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Excessive weight load | Sprain Back | - Use the correct capacity rollers/pallet truck to reduce the effort of each man when pushing the machine - Deploy adequate personnel | Moderate | Occasional | Medium | Use machine puller or forklift if possible to assist in pulling/pushing the machine | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | Minor | Occasional | Low | Good coordination | Supervisor on site |
|  |  | Obstruction to vision while moving |  | -To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck/rollers steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |