| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Manual Handling /Transporting Of Load No 49 Changi South Ave 2 (Medtronic Singapore Operation Pte Ltd) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 18 March 2015 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2012 | Next Review Date: | 14 May 2015 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Manual handling of equipment / load onto pallet truck | Improper lifting poster | Back injury | - Adopt proper technique when carrying out task - Buddy system when handling heavy equipment - Ensure there are enough manpower for the job | Moderate | Remote | Low |  | Team Leader |
|  |  | Accidental dropping of equipment | Foot injury | - PPE (etc.. safety shoe) issued and worn by all workers - If needed, use glove for firm grip on equipment - Ensure no unauthorised personnel within the working area | Moderate | Occasional | Medium | Use of mechanical lifting aids (etc.. manual lifter) if possible | Team Leader |
| 2 | Transportation of equipment to location | Equipment topper | Damage to Property | - Ensure equipment are place accordingly on the pallet truck - Workers to be alert and keep close lookout on route of travel from point to point | Moderate | Remote | Low | Slow and steady transportation speed | Team Leader |
|  |  | Obstruction to vision while moving | Bodily Injury / Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | Moderate | Remote | Low | Provision of spotter | Team Leader |
| 3 | Lift up equipment and put onto table stand | Improper lifting poster | Back Injury | - Adopt proper technique when carrying out task - Buddy system when handling heavy equipment - Ensure there are enough manpower for the job | Moderate | Remote | Low |  | Team Leader |