| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Transportation of machine to loading bay No 10 A.M.K Industrial Park 2 |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 18 Aug 2014 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2012 | Next Review Date: | 14 May 2015 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Jack up machine using manual / hydraulic jack | Machine topper | Bodily Injury & Damage to Property | - Ensure jack in good condition and able to lift weight of machine - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block to insert below machine | Major | Remote | Medium | Only skill and experience mover to control the jack handling | Supervisor on site |
| 2 | Insert rollers | Sharp edges of machine | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers to below machine with care - Use glove | Minor | Occasional | Low |  | Supervisor on site |
| 3 | Transportation of machine to hoisting bay | Rollers move out of position causing machine to topper | Bodily Injury & Damage to Property | - Ensure rollers are sitting at correct position on the machine base - Machine to be bolted onto rollers and secured using screw to prevent from rolling out - Ensure machine are stable on the rollers before moving - Workers to be alert and keep close lookout on route of travel from point to point and to ensure that rollers are not moving out of position | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passageway and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | Moderate | Occasional | Medium | Close supervision by supervisor | Supervisor on site |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the roller steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity pallet truck / rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | Moderate | Occasional | Medium | Use machine puller or forklift if possible | Supervisor on site |
| 4 | Pre-Lifting | Struck by falling objects | Bodily Injury & Damage to Property | - Certificate of testing by an approved person - Crane operation manual service manual made available - Regular through inspections carried out - Operator trained and competent person appointed - PPE (etc, safety helmet, safety shoe) provided and put on | Major | Remote | Medium | Lifting crew is pre-briefed before commencement of work | Lifting Supervisor on site |
| 5 | Site Preparation | Collapse or topple of crane due to uneven ground | Bodily Injury & Damage to Property | - Use steel plate / timber mats for ground support when required - Outrigger fully extended - Lifting area cordoned off | Major | Remote | Medium | Ensure lifting machine is on stable ground | Lifting Supervisor on site |
| 6 | Lifting Operation | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | Major | Remote | Medium | Ensure load, capacity of lifting arm and reach are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Riggers appointed and trained to carry out tasks - Lifting gears tested and approved by competent person - No defective gears allowed - Ensure that rigging of load as per manufacturing manual | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting Supervisor on site |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arm or Hand Injuries | - Keep hands off loose part of sling or lifting gears - Use glove | Moderate | Occasional | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | View obstruction or poor visibility, Load Swing | Bodily Injury & Damage to Property | - Lifting supervisor and the crane operator to check on the lifting path to ensure that it is clear from any obstruction - Appointed and trained signalman, riggers and lifting supervisors on site - Taglines should be used to control the swing motion - Never stand close or under the suspended load and away from possible swing path - Ensure area of operation clear of personnel - PPE (etc, safety helmet, safety shoe, glove) provided and put on | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor on site |