| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Shifting out / Hoisting machine No 78 Tuas South St 5 Enecal Pte Ltd |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 17 Feb 2016 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Lift up machine using crowbar / manual jack | Machine topper | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure crowbar / jack use are in good working condition - Ensure machine are lift up at the right angle of gravity - Lift up machine to 4” high from one side to another is carried out in stages with the use of hard wooden block | Major | Remote | Medium | Only skill and experience worker to control the crowbar/jack handling | Supervisor on site |
| 2 | Insert rollers / pallet truck | Sharp edges of machine | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers to below machine with care - Use glove | Minor | Occasional | Low |  | Supervisor on site |
| 3 | Transportation of machine to loading bay for packing | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured to machine base with bolts and nuts to prevent from rolling out - Ensure machine load is spread evenly on the rollers before being transport to loading area - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | Moderate | Occasional | Medium | Use manual puller or forklift if possible to assist in pulling/pushing the machine | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | Moderate | Occasional | Medium | Good coordination | Supervisor on site |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |
| 4 | Setting up of crane outriggers | Moving parts | Bodily Injury | - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation and to ensure all personnel is not in way of any moving parts of the crane) - All outrigger beams must be fully extended - Use steel plate / timber mats or any necessary enforcement for soft ground area - Use of suitable hand glove | Moderate | Remote | Low |  | Lorry loader operator |
| 5 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance to weight of machine - No defective gears allowed - Lifting zone to be barricade - Never stand close or under the load - PPE (etc.. safety helmet, safety shoe, glove) provided and put on | Major | Remote | Medium | While moving or slewing the crane, try not to jerk the load | Lifting Supervisor on site |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | Major | Remote | Medium | Ensure load, capacity of lifting arm and reach are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out task - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load are rig at it centre of gravity | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting Supervisor on site |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arm or Hand Injuries | - Ensure signalman give proper means of signal to crane operator - Keep hands off loose part of sling or lifting gears - Use glove | Moderate | Occasional | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | View obstruction or poor visibility, Load Swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisors on site - Use tag line for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (etc.. safety helmet, safety shoe, glove) provided and put on | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor on site |