| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd (Mazak) | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Moving/Hoisting of Machine No 2 Gul Circle Cameron (Singapore) Pte Ltd |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 05 Nov 2012 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2012 | Next Review Date: | 15 May 2015 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Jack up machine using hydraulic jack | Machine fall or slanting to one side | Bodily Injury & Damage to Property | - Use the correct capacity jack to lift up the machine - Workers to be alert when jacking up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 3” high is carried in stages with the use of hard wooden block to insert below machine | Major | Remote | Medium | Ensure area of operation clear of unauthorised personnel | Supervisor on site |
| 2 | Insert rollers | Sharp edges of machine | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers to below machine with care - Use glove fit for the purpose | Minor | Occasional | Low |  | Supervisor on site |
| 3 | Transportation of machine to loading area | Rollers move out of position causing machine to topper | Bodily Injury & Damage to Property | - Ensure machine are sitting at correct position on rollers - Ensure machine are stable on the rollers before moving - Machine to be bolted and secured onto rollers using screw to prevent from rolling out - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | Moderate | Occasional | Medium | Use machine puller or forklift if possible | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | Moderate | Occasional | Medium | Close supervision by on site supervisor | Supervisor on site |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close watch on route of travel - Ensure movement is guided - To alert the worker controlling the rollers steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |
| 4 | Using lorry crane to assist in pulling machine to loading area | Crane arm swing, sling snapped | Hitting people or objects | - Only certified crane operator to operate the crane - Signalman to assist in giving signal to crane operator - Certified lifting gears and lifting machine - Unauthorised personnel to stay clear from the working zone | Moderate | Occasional | Medium | Assignment of traffic controller | Supervisor on site |
| 5 | Pre-Lifting | Struck by falling objects | Bodily Injury & Damage to Property | - Certificate of testing by an approved person - Crane operations manual and service manual made available - Regular through inspections carried out - Operator trained and competent person appointed | Major | Remote | Medium | Close supervision and regular reminder by lifting supervisor | Lifting Supervisor on site |
| 6 | Site Preparation | Collapse or topple of crane due to uneven ground | Bodily Injury & Damage to Property | - Use steel plate/ timber mats for ground support when required - Outrigger extended - Lifting area cordoned off | Major | Remote | Medium | Ensure lifting machine is on stable ground | Lifting Supervisor on site |
| 7 | Insert wire to crane hook | Struck by crane hook | Bodily Injury & Damage to Property | - PPE (etc, safety helmet, safety shoe) provided and put on - Never stand close or under the hook - Signalman to signal to crane operator to lower down boom at slow speed - Use buddy system when inserting wire sling to crane hook | Major | Remote | Medium | Close supervision by supervisor | Lifting Supervisor on site |
| 8 | Lifting Operation 1 unit Mazak Integrex 70 CNC Lathe | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | Major | Remote | Medium | Ensure load, capacity of lifting arm and reach are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Riggers appointed and trained to carry out tasks - Ensure that rigging of load as per manufacturing manual only - Lifting gears tested and approved by competent person - No defective gears allowed | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting Supervisor on site |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arm or Hand Injuries | - Keep hands off loose part of sling or lifting gears - Gloves to be provided | Moderate | Occasional | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | View obstruction or poor visibility, Load Swing | Bodily Injury & Damage to Property | - Lifting supervisor and the crane operator to check on the lifting path to ensure that it is clear from any obstruction - Appointed and trained signalman, riggers and lifting supervisors on site - Taglines should be used to control the swing motion - Never stand close or under the suspended load and away from possible swing path - Ensure area of operation clear of personnel - PPE (etc, safety helmet, safety shoe, glove) provided and put on | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor on site |
| 9 | Transporting machine out from Cameron premises | Restricted driver visibility | Bodily Injury to workers and nearby people | - Low-bed equipped with side mirror and reverse warning alarm to cautioned nearby people - To exit by main gate - Driver advised to be careful when operating near blind spot, when moving forward and reversing | Moderate | Remote | Low | Assignment of traffic controller | Low-bed driver |