| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Wong Yew Fei (Team Leader) Lim Mong Kee (Team Leader) |  |
| Process/ Location: | Working at height to assemble crate No 15 Tuas View Place John Crane |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 04 Sep 2014 |  |  |  |  |
|  |  | Last Review Date: | # 04 Sep 2014 | Next Review Date: | 03 Sep 2017 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Nailing side of crate using ladder | Fall from ladder | Bodily Injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ladder must be fully extended and stand on level ground - Ensure buddy system-one to hold the ladder while another one is working on the ladder | Moderate | Remote | Low | Close supervision by supervisor | Supervisor on site |
|  | Covering top of crate | Falling from height | Fatality | - PPE (etc safety helmet, safety harness) provided and put on - Provision of alternative life line anchorage point, loop 2 meter webbing sling onto lorry crane main hook - Safety harness anchored and secured at all time on webbing sling eye to act as falling protection from height | Major | Remote | Medium | Supervisor to ensure all safety measure have been taken due to task being perform | Supervisor on site |