| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Working at Height No 20 Benoi Sector Shimano (S) Pte Ltd |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 02 Sep 2014 |  |  |  |  |
|  |  | Last Review Date: | # 02 Sep 2014 | Next Review Date: | 01 Sep 2017 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Working at height to un-crate case | Falling from height | Fatality | - Provision of alternative life line anchorage point, loop 2 meter webbing sling onto lorry crane main hook - Safety harness anchored and secured at all time onto webbing sling eye to act as falling protection from height | Major | Remote | Medium | Supervisor to ensure all safety measure have been taken due to task being perform | Supervisor on site |
| 2 | Working at height to assist in installation | Falling from height | Fatality | - To use ‘A’ frame ladder to work on which is unreachable from ground - Ladder must be fully extended and stand on ground level - Buddy system-one to hold the ladder while another is working on the ladder - Ensure personnel working at height is equipped with safety harness and secured to strong anchored point at all time | Major | Remote | Medium | Close supervision by site supervisor | Supervisor on site |