| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Mobile/Lorry Crane Operation Blk 3014A Ubi Rd 1 #05-07 (Samco Precision Pte Ltd) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 20th Feb 2017 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Pre-Lifting | Struck by falling objects | Bodily Injury & Damage to Property | - Certificate of testing by an approved person - Crane operations manual and service manual made available - Regular through inspections carried out - Operator trained and competent person appointed | Major | Remote | Medium | Lifting crew is pre-briefed before commencement of work | Lifting Supervisor / Crane Operator on site |
| 2 | Site Preparation | Collapse or topple of crane | Bodily Injury & Damage to Property | - Use steel plate / timber mats for ground support when required - Outrigger fully extended - Lifting area to be barricade, only relevant personnel are allowed to enter the lifting zone | Major | Remote | Medium | Ensure lifting machine is on firm and stable ground | Lifting Supervisor / Crane Operator on site |
| 3 | Lifting Operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry/mobile crane are valid - Riggers to ensure lifting gears use are in compliance to weight of machine - No defective gears allowed - Never stand close or under the load | Major | Remote | Medium | While moving or slewing the crane, try not to jerk the load | Lifting Supervisor on site |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | Major | Remote | Medium | Ensure loads, capacity of lifting arm and reach are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure load is rig at it centre of gravity | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting Supervisor on site |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to the operator - Glove to be provided and worn | Moderate | Occasional | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | Movement of load | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisors on site - Taglines should be used to control the swing motion - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (etc… safety helmet, safety shoe) provided and put on. | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor on site |
| 4 | Working at height on top platform | Falling from height | Bodily Injury or Death | - Workers working at height to wear safety helmet, safety harness - Ensure that safety harness use are in good working condition, properly strapped on and hook to strong anchorage point at all time - Practice good housekeeping | Major | Remote | Medium | Close supervision by site supervisor | Lifting Supervisor on site |
|  |  | Platform move or sway | Bodily Injury & Damage to Property | - Ensure platform is fully rested on the opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secure front left and right of platform to building railing to prevent platform swing outward | Major | Remote | Medium | Close supervision by site supervisor | Lifting Supervisor on site |
| 5 | Weather climate | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping / strike by lightning | - To stop work immediately if there is heavy rain / lightning strike | Major | Remote | Medium | Supervisor to ensure no activity during heavy rain | Supervisor on site |