| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Mobile Crane Operation / No 88 International Rd Hunting Energy Services Pte Ltd |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 05 June 2015 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Pre-Lifting | Struck by falling objects | Bodily Injury & Damage to Property | - Certificate of testing by an approved person - Crane operations manual and service manual made available - Regular through inspections carried out - Operator trained and competent person appointed - PPE (etc.., safety helmet, safety shoe) provided and put on | Major | Remote | Medium | Lifting crew is pre-briefed before commencement of work | Lifting Supervisor / Crane Operator |
| 2 | Setting up of outriggers | Moving parts | Bodily Injury | - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation and ensure all personnel is not in-way of any parts of the crane) - All outrigger beams must be fully extended - Use steel plate / timber mats or any necessary enforcement for soft ground area - Lifting area cordoned off | Major | Remote | Medium | Ensure lifting machine is on firm and stable ground | Lifting Supervisor on site |
| 3 | Hoisting of platform to level 2 opening | Incorrect signal to crane operator | Damage to Property | - Ensure that signalman possesses relevant skill and trained to carry out tasks - To adequately provide and ensure proper and clear signalling is given to the crane operator | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor on site |
| 4 | Securing of platform using 100 ton mobile crane | Collision of crane boom | Bodily Injury & Damage to Property | - Lifting supervisor and crane operator to check on the lifting path to ensure sufficient turning angle in between the crane - To adequately provide and ensure proper and clear signal is given to the crane operator - Operator trained and competent person appointed | Major | Remote | Medium | Never stand close or under the suspended load and away from possible swing path | Lifting Supervisor on site |
|  |  | Improper tie back | Topper of platform | - Ensure platform is fully rested on the opening slab - Ensure platform is tied back and secured to a competent point - Rigger to ensure that lifting gears use are valid and in compliance to the weight of the platform and machine | Major | Remote | Medium | Ensure platform is stabilise effectively | Lifting Supervisor on site |
|  |  | Falling from height | Bodily Injury / Fatality | - Ensure personnel’s entering working platform is equipped with safety helmet, safety harness - Ensure that safety harness are properly strapped on and hook to strong anchorage point at all time | Major | Remote | Medium | Close supervision by site supervisor | Lifting/Site Supervisor |
| 5 | Lifting Operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (etc… safety shoe, safety helmet) provided and put on | Major | Remote | Medium | While moving or slewing the crane, try not to jerk the load | Lifting Supervisor on site |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | Major | Remote | Medium | Ensure loads, capacity of lifting arm and reach are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out task - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load are rig at it centre of gravity | Major | Remote | Medium | Communication through hand signal by Eric Wong personnel only | Lifting Supervisor on site |
|  |  | Hands caught by or in between loose parts of sling or lifting gears | Arm or Hand Injury | -Keep hands off loose part of sling or lifting gears - Use glove | Moderate | Occasional | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | View obstruction or poor visibility, Load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisors on site - Clear obstruction and unauthorised personnel in way of lifting path - Tag lines should be used to control the load from swinging so as to prevent bodily contact with load - Never stand close or under the suspended load and away from possible swing path | Major | Remote | Medium | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | Lifting Supervisor / Signalman / Crane Operator |
| 6 | Weather climate | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping, strike by lightning | - To stop work immediately if there is heavy rain and lightning strike | Major | Remote | Medium | Supervisor to ensure no activity during heavy rain | Supervisor on site |