| Date | Description of Change |  | Rev No |
| --- | --- | --- | --- |
| 1st July 2023 | Initial release |  | 00 |
| Prepared by: Kevin Wong |  | Reviewed and Approved by: Eric Wong |  |
| Signature: Designation: Project Manager Date: 01st July 2023 |  | Signature: Designation: Managing Director Date: 01st July 2023 |  |

- Objective

To identify the hazard associated with lifting operation and their safe use so as to protect our workers and neighbours from incident using crane (including excavator).

- Scope

This procedure is applicable to all lifting operations, if necessary. Where work is carried out at the client’s premises and the client has a SWP, the client’s SWP shall be followed.

- Related documents/reference

  - Workplace Safety & Health Act

  - WSH (Construction) Regulations

  - WSH (Operation of Crane) Regulations

  - Code of Practice on Safe Lifting Operations in the Workplaces

  - SS 536	Safe Use of Mobile Cranes

  - SS 617 Code of Practice for Lifting of Persons In Work Platform Suspended from Cranes

  - SWP-SLA	Safe Use of Lifting Appliance & Gear

  - SWP-SLC	Safe Use of Lorry Crane

  - Annex 1	Hazards Related to Lifting Operation

- Responsibility

  - Safety Supervisor (STS) / Site Supervisor (SS) shall be responsible for ensuring that this procedure is complied with while on site.

- Procedure

  - Lifting Team shall be appointed with certificates maintained including certificate of lifting equipment.

  - Lifting Supervisor (LS) shall conduct pre-lifting discussion with his team using Toolbox Meeting minutes to record such discussion.

  - No lifting operation shall be performed when the load (stationary or otherwise) exceeds the safe working load.

  - The LS must verify that the strength of the lifting gears and equipment are adequate and their certified safe working load is in line with the load they bear.

  - The lifting operation area and other areas with potential hazard / danger must be cordon off to prevent access for all unauthorized personnel to danger areas. Only authorized persons and personnel involved in the lift are allowed in the lifting operation area where applicable.

  - Safety measures shall be taken to address any potential hazards and/or danger that may arise out of the lifting operation.

  - Weather forecasts shall be monitored before and during lifting work.

  - The STS shall inform LS prior to commencing lifting work whenever the wind speeds are high.

  - The Operator shall be able to observe the load continually during lifting.

  - The loads shall be raised vertically to avoid swinging during lifting.

  - The load shall be controlled by means of tag lines to prevent the load from turning or swinging.

  - Hoisting or lowering loose materials such as stones, bricks tiles, slates, bolts and nuts, shall be done in covered containers, receptacles or cargo net, and to be secured or enclosed firmly.

  - Tying or guying to existing structure or constructions is not permitted unless otherwise approval is given by the STS.

  - Lifting and crane operation, rigging and signaling shall be performed by competent, trained and authorized persons.

  - Rigger and Signalman shall wear distinctively colored safety helmets or other clearly identifiable clothing.

  - Signals shall be specified. Two-way radio communications shall be advocated when necessary.

  - Only the person assigned is allowed to give signals and signs to the crane operators.

  - All lifting gear must be identifiable and checked before and after use.

  - Lifting beam or spreader-bar or wire ropes shall be used for hoisting long materials, to space the sling legs for proper balance.

  - When a load is suspended at two or more points with slings, the eyes of the lifting legs or the slings shall be shackled together and placed on the hook or shackled directly to the hoisting block, ball or balance beam, or placed on the lifting hook without shackles if the hook is provided with safety features for such use.

  - Hoisting of personnel to height by crane is strictly prohibited, unless using method approved by Ministry of Manpower.

  - Crane crossing or working at location with heavy equipment and/or loads where underground cables and/or pipe line are located, adequate protection such as mats or steel plates shall be used to prevent damage.

  - Emergency during Lifting Operations

Dependent on the alarm situation and the lifting operation, the following measures must be taken during alarm:

    - The load must be lowered in case of the alarm situation.

    - The adjacent area must be marked out clearly. This area must be included in the plan before lifting started.

    - The motors of the hoisting equipment must be stopped immediately.

    - After the 'ALL SAFE' signal have been given, the work area, lifting equipment and the load must be re-inspected to verify whether the work conditions have not been changed during the alarm.

Annex 1	Hazards Related to Lifting Operation

| Activity | Aspect / Hazard | Precautions |
| --- | --- | --- |
| Lifting of load | - Crane topples | - Ensure that that the crane access is firm, if applicable - Do not overload the crane - Know the load to be lifted - Do not drag the load - Do not carry the load and travel with the crane - Conduct Crane Checklist and Pre-lifting Checklist - Outrigger must be fully extended and set on firm foundation |
|  | - Hit by lifted object | - Tag line to be used to minimize physical contact with the lifted object - Do not stand under suspended load - Cargo net to be use if necessary - Avoid swinging the load over area where there are workers working underneath - Load lifted must be properly rigged |
|  | - Hit by or contact with machinery | - Avoid the operating radius of the machine - Give way to the traveling machine |
|  | - Hit people | 1) Barricade off work area to prevent unauthorized entry 2) Lifting in operation signage to warn others that lifting work are in progress |